AILL v1.1 Codebook Reference

Acoustic Inter-agent Linguistic Link — Complete Domain Codebook Registry

This document contains the complete Level 1 domain codebook definitions for AILL v1.1. The seven standard domain codebooks define 562 semantic entries organized into functional sections. Combined with the 256-entry Base Codebook (Level 0), AILL provides 818 pre-defined concepts for inter-agent communication.

Codebook Registry Summary

| ID | Codebook | Domain | Entries |
| --- | --- | --- | --- |
| 0x01 | NAV-1 | Navigation and spatial positioning | 104 |
| 0x02 | PERCEPT-1 | Visual and sensor perception | 104 |
| 0x03 | MANIP-1 | Robotic manipulation and grasping | 75 |
| 0x04 | COMM-1 | Inter-agent communication and social protocols | 63 |
| 0x05 | DIAG-1 | Diagnostic and system health reporting | 86 |
| 0x06 | PLAN-1 | Task planning and goal management | 67 |
| 0x07 | SAFETY-1 | Safety, emergency, and regulatory compliance | 63 |
|  | TOTAL | All Level 1 domain codebooks | 562 |

# NAV-1 — Navigation and spatial positioning

Registry ID: 0x01  |  104 entries

| Code | Mnemonic | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| 0x0000 | POSITION_3D | ARRAY<FLOAT32,3> | m | 3D position (x, y, z) |
| 0x0001 | POSITION_2D | ARRAY<FLOAT32,2> | m | 2D position (x, y) |
| 0x0002 | HEADING | FLOAT32 | rad | Heading angle from North |
| 0x0003 | ORIENTATION_QUAT | ARRAY<FLOAT32,4> |  | Quaternion (w, x, y, z) |
| 0x0004 | ORIENTATION_EULER | ARRAY<FLOAT32,3> | rad | Euler angles (roll, pitch, yaw) |
| 0x0005 | VELOCITY_3D | ARRAY<FLOAT32,3> | m/s | Linear velocity vector |
| 0x0006 | VELOCITY_SCALAR | FLOAT32 | m/s | Scalar speed |
| 0x0007 | ANGULAR_VEL | ARRAY<FLOAT32,3> | rad/s | Angular velocity |
| 0x0008 | ACCELERATION_3D | ARRAY<FLOAT32,3> | m/s^2 | Linear acceleration |
| 0x0009 | POSE_6DOF | STRUCT{pos,orient} |  | Full 6DOF pose |
| 0x000A | LATITUDE | FLOAT64 | deg | WGS84 latitude |
| 0x000B | LONGITUDE | FLOAT64 | deg | WGS84 longitude |
| 0x000C | ALTITUDE_MSL | FLOAT32 | m | Altitude above mean sea level |
| 0x000D | ALTITUDE_AGL | FLOAT32 | m | Altitude above ground level |
| 0x000E | GPS_FIX | STRUCT |  | Complete GPS fix record |
| 0x000F | COORDINATE_FRAME | UINT8 |  | Coord frame ID |
| 0x0030 | WAYPOINT | STRUCT{id,pos,rad} |  | Named waypoint |
| 0x0031 | WAYPOINT_ID | UINT16 |  | Waypoint identifier |
| 0x0032 | PATH | LIST<WAYPOINT> |  | Ordered waypoint sequence |
| 0x0033 | PATH_SEGMENT | STRUCT |  | Segment with curvature |
| 0x0034 | CURRENT_WAYPOINT | UINT16 |  | Current target waypoint index |
| 0x0035 | DISTANCE_TO_WP | FLOAT32 | m | Distance to current waypoint |
| 0x0036 | ETA | FLOAT32 | s | Estimated time of arrival |
| 0x0037 | PATH_COMPLETE | BOOL |  | Path completion flag |
| 0x0038 | PATH_DEVIATION | FLOAT32 | m | Cross-track error |
| 0x0039 | GEOFENCE | LIST<POSITION_2D> |  | Restricted area polygon |
| 0x003A | GEOFENCE_STATUS | UINT8 |  | Geofence relation status |
| 0x003B | HOME_POSITION | POSITION_3D | m | Designated home position |
| 0x0060 | OBSTACLE | STRUCT |  | Detected obstacle |
| 0x0061 | OBSTACLE_TYPE | UINT8 |  | Obstacle classification |
| 0x0062 | OBSTACLE_SIZE | ARRAY<FLOAT32,3> | m | Bounding box dimensions |
| 0x0063 | OBSTACLE_LIST | LIST<OBSTACLE> |  | Collection of obstacles |
| 0x0064 | CLEARANCE | FLOAT32 | m | Min clearance to nearest obstacle |
| 0x0065 | COLLISION_RISK | FLOAT16 |  | Collision probability 0.0-1.0 |
| 0x0066 | TERRAIN_TYPE | UINT8 |  | Surface type code |
| 0x0067 | SLOPE_ANGLE | FLOAT16 | rad | Ground slope |
| 0x0068 | VISIBILITY | FLOAT32 | m | Visibility range |
| 0x0069 | OCCUPANCY_GRID | STRUCT |  | 2D occupancy grid map |
| 0x0090 | GOTO | POSITION_3D | m | Navigate to position |
| 0x0091 | GOTO_WAYPOINT | UINT16 |  | Navigate to waypoint ID |
| 0x0092 | FOLLOW_PATH | PATH |  | Execute path |
| 0x0093 | STOP | NONE |  | Halt all movement |
| 0x0094 | HOLD_POSITION | NONE |  | Station-keeping |
| 0x0095 | SET_VELOCITY | VELOCITY_3D | m/s | Set desired velocity |
| 0x0096 | SET_HEADING | FLOAT32 | rad | Turn to heading |
| 0x0097 | ORBIT | STRUCT |  | Orbit a point |
| 0x0098 | FOLLOW_AGENT | STRUCT{uuid,dist} |  | Follow another agent |
| 0x0099 | RETURN_HOME | NONE |  | Navigate to home |
| 0x009A | AVOID | STRUCT{pos,radius} |  | Add exclusion zone |
| 0x009B | FORMATION | STRUCT{type,slot} |  | Join formation |
| 0x00C0 | MAP_ORIGIN | POSITION_3D | m | Origin of the local map frame |
| 0x00C1 | MAP_RESOLUTION | FLOAT32 | m | Grid cell size |
| 0x00C2 | MAP_DIMENSIONS | ARRAY<UINT16,3> |  | Grid dimensions (nx, ny, nz) |
| 0x00C3 | MAP_UPDATE | STRUCT{region,cells} |  | Incremental map patch |
| 0x00C4 | MAP_VERSION | UINT32 |  | Map revision counter |
| 0x00C5 | LOCALIZATION_CONF | FLOAT16 |  | Localization confidence 0.0-1.0 |
| 0x00C6 | POSE_COVARIANCE | ARRAY<FLOAT32,36> |  | 6x6 pose uncertainty covariance matrix |
| 0x00C7 | LANDMARK | STRUCT{id,pos,desc} |  | Recognized environmental landmark |
| 0x00C8 | LANDMARK_LIST | LIST<LANDMARK> |  | Collection of observed landmarks |
| 0x00C9 | LOOP_CLOSURE | STRUCT{from,to,tf} |  | Loop closure detection with transform |
| 0x00CA | RELOCALIZE | NONE |  | Trigger relocalization procedure |
| 0x00CB | LOCALIZATION_MODE | UINT8 |  | 0=SLAM, 1=known_map, 2=GPS_primary, 3=visual_odom, 4=dead_reckoning |
| 0x00CC | ODOMETRY_DRIFT | FLOAT32 | m | Estimated cumulative drift from odometry |
| 0x00CD | VISUAL_FEATURES | UINT16 |  | Number of tracked visual features |
| 0x00CE | MAP_MERGE_REQ | STRUCT{agent,hash} |  | Request to merge map from another agent |
| 0x00CF | MAP_MERGE_ACK | STRUCT{agent,tf} |  | Acknowledge merge with alignment transform |
| 0x00F0 | TRANSFORM_3D | STRUCT{rot,trans} |  | Rigid body transform (rotation matrix + translation) |
| 0x00F1 | TRANSFORM_QUAT | STRUCT{quat,trans} |  | Quaternion-based rigid body transform |
| 0x00F2 | FRAME_PARENT | UINT8 |  | Parent frame ID in transform tree |
| 0x00F3 | FRAME_CHILD | UINT8 |  | Child frame ID in transform tree |
| 0x00F4 | TF_TREE | LIST<STRUCT{parent,child,tf}> |  | Complete transform tree snapshot |
| 0x00F5 | TF_LOOKUP | STRUCT{from,to} |  | Request transform between two frames |
| 0x00F6 | TF_RESULT | TRANSFORM_QUAT |  | Result of a transform lookup |
| 0x00F7 | DATUM_WGS84 | STRUCT{lat,lon,alt} |  | WGS84 datum point for local frame |
| 0x00F8 | UTM_ZONE | STRUCT{zone,band} |  | UTM zone number and latitude band |
| 0x00F9 | MAGNETIC_DECLINATION | FLOAT16 | rad | Local magnetic declination |
| 0x0110 | SWARM_CENTER | POSITION_3D | m | Centroid of all agents in swarm |
| 0x0111 | SWARM_RADIUS | FLOAT32 | m | Bounding radius of swarm |
| 0x0112 | AGENT_POSITIONS | LIST<STRUCT{uuid,pos}> |  | Positions of all known agents |
| 0x0113 | SEPARATION_DIST | FLOAT32 | m | Minimum inter-agent separation distance |
| 0x0114 | COLLISION_ALERT | STRUCT{agent,ttc} |  | Inter-agent collision alert with time-to-collision |
| 0x0115 | ZONE_CLAIM | STRUCT{agent,polygon} |  | Agent claims exclusive operating zone |
| 0x0116 | ZONE_RELEASE | STRUCT{agent,zone_id} |  | Agent releases zone claim |
| 0x0117 | RENDEZVOUS_POINT | STRUCT{pos,time} |  | Designated meeting point with time |
| 0x0118 | CONVOY_JOIN | STRUCT{leader,pos,slot} |  | Join a convoy behind specified leader |
| 0x0119 | CONVOY_LEAVE | NONE |  | Depart from current convoy |
| 0x011A | COVERAGE_MAP | STRUCT{grid,visited} |  | Coverage completion map for area search |
| 0x011B | ASSIGN_SECTOR | STRUCT{agent,polygon} |  | Assign search sector to agent |
| 0x011C | SECTOR_COMPLETE | STRUCT{agent,sector_id} |  | Report sector search complete |
| 0x011D | RELATIVE_BEARING | STRUCT{agent,bearing,range} |  | Bearing and range to another agent |
| 0x011E | FORMATION_OFFSET | STRUCT{slot,offset} |  | Position offset within formation for assigned slot |
| 0x011F | TRAFFIC_DECONFLICT | STRUCT{agent,corridor} |  | Corridor assignment for traffic deconfliction |
| 0x0140 | SPLINE_PATH | STRUCT{ctrl_pts,order} |  | Spline-based smooth path (control points + order) |
| 0x0141 | DUBINS_PATH | STRUCT{start,end,radius} |  | Dubins path for non-holonomic vehicles |
| 0x0142 | VELOCITY_PROFILE | LIST<STRUCT{dist,vel}> |  | Speed profile along path |
| 0x0143 | ALTITUDE_PROFILE | LIST<STRUCT{dist,alt}> |  | Altitude profile along path |
| 0x0144 | NO_FLY_ZONE | STRUCT{polygon,floor,ceiling} |  | 3D restricted airspace volume |
| 0x0145 | DYNAMIC_OBSTACLE | STRUCT{id,pos,vel,pred_path} |  | Obstacle with predicted trajectory |
| 0x0146 | REPLAN_TRIGGER | UINT8 |  | 0=obstacle, 1=path_blocked, 2=priority_change, 3=battery_low, 4=weather |
| 0x0147 | SEARCH_PATTERN | UINT8 |  | 0=lawnmower, 1=spiral, 2=sector, 3=random_walk, 4=levy_flight |
| 0x0148 | LOITER | STRUCT{center,radius,alt,duration} |  | Loiter (circle) at location for duration |
| 0x0149 | LANDING_ZONE | STRUCT{pos,heading,slope,clear} |  | Designated landing area with surface info |
| 0x014A | TAKEOFF | STRUCT{alt} |  | Takeoff to specified altitude |
| 0x014B | LAND | STRUCT{zone_id} |  | Land at designated landing zone |

# PERCEPT-1 — Visual and sensor perception

Registry ID: 0x02  |  104 entries

| Code | Mnemonic | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| 0x0000 | DETECTED_OBJECT | STRUCT |  | Detected object with properties |
| 0x0001 | OBJECT_CLASS | UINT16 |  | Object class from taxonomy |
| 0x0002 | OBJECT_CONFIDENCE | FLOAT16 |  | Detection confidence 0.0-1.0 |
| 0x0003 | BOUNDING_BOX_2D | ARRAY<FLOAT32,4> | px | 2D bbox (x, y, width, height) |
| 0x0004 | BOUNDING_BOX_3D | STRUCT | m | 3D bbox (center, dimensions, orientation) |
| 0x0005 | OBJECT_POSITION | ARRAY<FLOAT32,3> | m | Object centroid in 3D |
| 0x0006 | OBJECT_VELOCITY | ARRAY<FLOAT32,3> | m/s | Object velocity estimate |
| 0x0007 | OBJECT_ID | UINT32 |  | Tracking ID (persistent across frames) |
| 0x0008 | OBJECT_LIST | LIST<DETECTED_OBJECT> |  | Collection of detections |
| 0x0009 | SEGMENTATION_MASK | BYTES |  | Run-length encoded pixel mask |
| 0x000A | KEYPOINT | ARRAY<FLOAT32,3> | px | 2D keypoint (x, y, confidence) |
| 0x000B | KEYPOINT_SET | LIST<KEYPOINT> |  | Named set of keypoints (skeleton) |
| 0x000C | OBJECT_LABEL | STRING |  | Human-readable label |
| 0x0030 | ABOVE | NONE |  | Spatial: A is above B |
| 0x0031 | BELOW | NONE |  | Spatial: A is below B |
| 0x0032 | LEFT_OF | NONE |  | Spatial: A is left of B |
| 0x0033 | RIGHT_OF | NONE |  | Spatial: A is right of B |
| 0x0034 | IN_FRONT_OF | NONE |  | Spatial: A is in front of B |
| 0x0035 | BEHIND | NONE |  | Spatial: A is behind B |
| 0x0036 | INSIDE | NONE |  | Spatial: A is inside B |
| 0x0037 | OUTSIDE | NONE |  | Spatial: A is outside B |
| 0x0038 | ADJACENT | NONE |  | Spatial: A is adjacent to B |
| 0x0039 | FAR_FROM | NONE |  | Spatial: A is far from B |
| 0x003A | NEAR | NONE |  | Spatial: A is near B |
| 0x003B | ON_TOP_OF | NONE |  | Spatial: A is resting on B |
| 0x003C | ATTACHED_TO | NONE |  | Spatial: A is physically attached to B |
| 0x0050 | COLOR_RGB | ARRAY<UINT8,3> |  | Color as (R, G, B) |
| 0x0051 | COLOR_NAME | UINT8 |  | Named color index |
| 0x0052 | TEXTURE | UINT8 |  | Texture class (smooth, rough, etc.) |
| 0x0053 | MATERIAL | UINT8 |  | Material class (metal, wood, etc.) |
| 0x0054 | SHAPE | UINT8 |  | Shape class (sphere, cube, etc.) |
| 0x0055 | SIZE_RELATIVE | UINT8 |  | Relative size (tiny, small, medium, large, huge) |
| 0x0056 | BRIGHTNESS | FLOAT16 | lux | Measured brightness |
| 0x0057 | TRANSPARENCY | FLOAT16 |  | Transparency 0.0-1.0 |
| 0x0070 | LIDAR_SCAN | LIST<ARRAY<FLOAT32,3>> | m | Point cloud from LiDAR |
| 0x0071 | DEPTH_MAP | STRUCT{w,h,data} | m | Depth image |
| 0x0072 | CAMERA_INTRINSICS | STRUCT |  | Camera calibration matrix |
| 0x0073 | CAMERA_EXTRINSICS | STRUCT |  | Camera pose |
| 0x0074 | IMAGE_EMBEDDING | ARRAY<FLOAT16,N> |  | Feature embedding vector |
| 0x0075 | AUDIO_LEVEL | FLOAT16 | dB | Ambient audio level |
| 0x0076 | TEMPERATURE | FLOAT16 | K | Measured temperature |
| 0x0077 | HUMIDITY | FLOAT16 | % | Relative humidity |
| 0x0078 | PRESSURE | FLOAT32 | Pa | Atmospheric pressure |
| 0x0079 | IMU_DATA | STRUCT{accel,gyro,mag} |  | Inertial measurement unit |
| 0x0090 | SCENE_GRAPH | LIST<STRUCT{subj,rel,obj}> |  | Scene graph: subject-relation-object triples |
| 0x0091 | ROOM_TYPE | UINT8 |  | 0=unknown, 1=corridor, 2=room, 3=outdoor, 4=stairwell, 5=elevator, 6=garage, 7=warehouse |
| 0x0092 | FLOOR_LEVEL | INT8 |  | Building floor number (-N for basement) |
| 0x0093 | SURFACE_NORMAL | ARRAY<FLOAT32,3> |  | Dominant surface normal vector |
| 0x0094 | PLANE_SEGMENT | STRUCT{normal,d,bounds} |  | Detected planar surface segment |
| 0x0095 | PLANE_LIST | LIST<PLANE_SEGMENT> |  | All detected planar surfaces |
| 0x0096 | SEMANTIC_LABEL | STRUCT{region,class,conf} |  | Semantic segmentation label for a region |
| 0x0097 | SCENE_COMPLEXITY | FLOAT16 |  | Scene complexity score 0.0-1.0 |
| 0x0098 | CLUTTER_DENSITY | FLOAT16 |  | Object density per cubic meter |
| 0x0099 | TRAVERSABILITY | FLOAT16 |  | Surface traversability score 0.0-1.0 |
| 0x009A | DOOR_STATE | STRUCT{pos,state} |  | Door: 0=closed, 1=open, 2=ajar, 3=locked |
| 0x009B | OPENING | STRUCT{pos,width,height} | m | Passable opening (doorway, gap) |
| 0x009C | STAIRS | STRUCT{pos,direction,count} |  | Detected staircase with step count |
| 0x009D | RAMP | STRUCT{pos,slope,width} |  | Detected ramp or incline |
| 0x009E | SIGN_TEXT | STRUCT{pos,text,lang} |  | Detected and OCR'd sign text |
| 0x009F | QR_CODE | STRUCT{pos,data} |  | Detected QR code with decoded data |
| 0x00B0 | MOTION_DETECTED | STRUCT{region,magnitude} |  | Motion detected in field of view |
| 0x00B1 | OBJECT_APPEARED | STRUCT{id,class,pos} |  | New object entered field of view |
| 0x00B2 | OBJECT_DISAPPEARED | STRUCT{id,last_pos} |  | Tracked object left field of view |
| 0x00B3 | OBJECT_STOPPED | STRUCT{id,pos,duration} |  | Moving object has stopped |
| 0x00B4 | OBJECT_PICKED_UP | STRUCT{id,agent} |  | Object was picked up |
| 0x00B5 | OBJECT_PLACED | STRUCT{id,surface} |  | Object was placed on surface |
| 0x00B6 | GESTURE_DETECTED | STRUCT{type,agent,conf} |  | Human gesture recognized |
| 0x00B7 | GESTURE_TYPE | UINT8 |  | 0=wave, 1=point, 2=stop, 3=come, 4=thumbs_up, 5=thumbs_down, 6=nod, 7=shake_head |
| 0x00B8 | FACE_DETECTED | STRUCT{bbox,landmarks,id} |  | Detected human face with optional ID |
| 0x00B9 | FACE_EXPRESSION | UINT8 |  | 0=neutral, 1=happy, 2=sad, 3=angry, 4=surprised, 5=fearful, 6=disgusted |
| 0x00BA | PERSON_POSE | LIST<KEYPOINT> |  | Full body skeleton keypoints |
| 0x00BB | ACTIVITY_CLASS | UINT8 |  | 0=standing, 1=walking, 2=running, 3=sitting, 4=lying, 5=falling, 6=working, 7=waving |
| 0x00BC | CROWD_DENSITY | FLOAT16 | 1/m^2 | People per square meter in region |
| 0x00BD | ANOMALY_DETECTED | STRUCT{type,pos,conf} |  | Anomalous event or state detected |
| 0x00BE | LIGHT_CHANGE | STRUCT{before,after} | lux | Significant illumination change |
| 0x00BF | OCCLUSION | STRUCT{obj_id,pct} |  | Object partially occluded (percent hidden) |
| 0x00D0 | SOUND_EVENT | STRUCT{class,dir,level} |  | Detected sound event |
| 0x00D1 | SOUND_CLASS | UINT8 |  | 0=speech, 1=alarm, 2=impact, 3=engine, 4=music, 5=animal, 6=footsteps, 7=glass_break |
| 0x00D2 | SOUND_DIRECTION | ARRAY<FLOAT32,2> | rad | Azimuth and elevation of sound source |
| 0x00D3 | SOUND_LEVEL | FLOAT16 | dB_SPL | Sound pressure level |
| 0x00D4 | SPEECH_DETECTED | STRUCT{dir,lang,dur} |  | Speech activity detected |
| 0x00D5 | SPEECH_TEXT | STRUCT{text,lang,conf} |  | Speech-to-text transcription result |
| 0x00D6 | SPEAKER_ID | STRUCT{uuid,conf} |  | Identified speaker (voice print match) |
| 0x00D7 | AMBIENT_NOISE | FLOAT16 | dB_SPL | Background noise floor level |
| 0x00D8 | ALARM_ACTIVE | STRUCT{type,pos,level} |  | Active alarm detected (fire, security, etc.) |
| 0x00E0 | CONTACT_DETECTED | STRUCT{pos,normal,force} |  | Physical contact detected |
| 0x00E1 | CONTACT_FORCE | ARRAY<FLOAT32,3> | N | Contact force vector |
| 0x00E2 | CONTACT_AREA | FLOAT32 | m^2 | Estimated contact patch area |
| 0x00E3 | SURFACE_FRICTION | FLOAT16 |  | Estimated surface friction coefficient |
| 0x00E4 | VIBRATION | STRUCT{freq,amplitude} |  | Detected vibration (frequency and amplitude) |
| 0x00E5 | LOAD_CELL | STRUCT{id,force} | N | Load cell reading |
| 0x00E6 | TORQUE_SENSOR | STRUCT{id,torque} | Nm | Torque sensor reading |
| 0x00E7 | SLIP_DETECTED | STRUCT{gripper,obj} |  | Object slippage detected at gripper |
| 0x00E8 | PROXIMITY_SENSOR | STRUCT{id,range} | m | Proximity sensor reading |
| 0x00F0 | GAS_CONCENTRATION | STRUCT{gas,ppm} | ppm | Gas concentration (CO, CO2, CH4, etc.) |
| 0x00F1 | RADIATION_LEVEL | FLOAT32 | uSv/h | Radiation dose rate |
| 0x00F2 | WIND_SPEED | FLOAT32 | m/s | Measured wind speed |
| 0x00F3 | WIND_DIRECTION | FLOAT32 | rad | Wind direction (from) |
| 0x00F4 | RAIN_RATE | FLOAT16 | mm/h | Precipitation rate |
| 0x00F5 | UV_INDEX | FLOAT16 |  | Ultraviolet radiation index |
| 0x00F6 | AIR_QUALITY_INDEX | UINT16 |  | Air quality index (0-500) |
| 0x00F7 | DUST_DENSITY | FLOAT32 | ug/m^3 | Particulate matter concentration |
| 0x00F8 | MAGNETIC_FIELD | ARRAY<FLOAT32,3> | uT | Local magnetic field vector |
| 0x00F9 | LIGHT_SPECTRUM | STRUCT{wavelengths,intensities} |  | Spectral light measurement |

# MANIP-1 — Robotic manipulation and grasping

Registry ID: 0x03  |  75 entries

| Code | Mnemonic | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| 0x0000 | GRIPPER_STATE | UINT8 |  | 0=open, 1=closing, 2=closed, 3=opening, 4=holding, 5=error |
| 0x0001 | GRIPPER_WIDTH | FLOAT32 | m | Current gripper aperture width |
| 0x0002 | GRIPPER_FORCE | FLOAT32 | N | Current gripper force |
| 0x0003 | GRIPPER_SET_WIDTH | FLOAT32 | m | Commanded gripper width |
| 0x0004 | GRIPPER_SET_FORCE | FLOAT32 | N | Commanded gripper force limit |
| 0x0005 | TOOL_TYPE | UINT8 |  | 0=parallel_jaw, 1=vacuum, 2=magnetic, 3=soft, 4=finger_3, 5=hook, 6=scoop, 7=custom |
| 0x0006 | TOOL_CENTER_POINT | ARRAY<FLOAT32,3> | m | Tool center point (TCP) in end-effector frame |
| 0x0007 | TOOL_CHANGE_REQ | UINT8 |  | Request tool change to specified tool type |
| 0x0008 | TOOL_CHANGE_ACK | UINT8 |  | Tool change completed |
| 0x0009 | SUCTION_PRESSURE | FLOAT32 | Pa | Vacuum gripper suction pressure |
| 0x000A | SUCTION_STATUS | UINT8 |  | 0=off, 1=engaged, 2=leak, 3=lost_seal |
| 0x000B | FINGER_POSITIONS | LIST<FLOAT32> | rad | Per-finger joint positions |
| 0x000C | FINGER_FORCES | LIST<FLOAT32> | N | Per-finger contact forces |
| 0x000D | TACTILE_ARRAY | STRUCT{rows,cols,data} | Pa | Tactile sensor pad readings |
| 0x0020 | JOINT_POSITIONS | LIST<FLOAT32> | rad | All joint angles |
| 0x0021 | JOINT_VELOCITIES | LIST<FLOAT32> | rad/s | All joint angular velocities |
| 0x0022 | JOINT_TORQUES | LIST<FLOAT32> | Nm | All joint torques |
| 0x0023 | JOINT_LIMITS | LIST<STRUCT{min,max}> | rad | Joint angle limits |
| 0x0024 | JOINT_TARGET | LIST<FLOAT32> | rad | Commanded joint positions |
| 0x0025 | JOINT_TRAJECTORY | LIST<STRUCT{time,positions}> |  | Time-parameterized joint trajectory |
| 0x0026 | JOINT_IMPEDANCE | STRUCT{stiffness,damping} |  | Joint impedance parameters |
| 0x0027 | DOF_COUNT | UINT8 |  | Number of degrees of freedom |
| 0x0028 | DH_PARAMETERS | LIST<STRUCT{a,alpha,d,theta}> |  | Denavit-Hartenberg kinematic parameters |
| 0x0029 | SINGULARITY_PROXIMITY | FLOAT16 |  | Distance to kinematic singularity 0.0-1.0 |
| 0x0040 | EE_POSE | STRUCT{pos,orient} |  | End-effector pose in base frame |
| 0x0041 | EE_VELOCITY | STRUCT{linear,angular} |  | End-effector twist (linear + angular velocity) |
| 0x0042 | EE_WRENCH | STRUCT{force,torque} |  | End-effector wrench (force + torque) |
| 0x0043 | CARTESIAN_TARGET | STRUCT{pos,orient} |  | Commanded end-effector pose |
| 0x0044 | CARTESIAN_PATH | LIST<STRUCT{pos,orient,time}> |  | Cartesian trajectory waypoints |
| 0x0045 | WORKSPACE_LIMIT | STRUCT{min,max} | m | Reachable workspace bounding box |
| 0x0046 | COMPLIANCE_FRAME | STRUCT{pos,orient} |  | Reference frame for compliance control |
| 0x0047 | IMPEDANCE_PARAMS | STRUCT{mass,damping,stiffness} |  | Cartesian impedance parameters |
| 0x0048 | FORCE_THRESHOLD | STRUCT{force,torque} |  | Force/torque thresholds for safety stop |
| 0x0060 | GRASP_POSE | STRUCT{pos,orient,width} |  | Planned grasp pose |
| 0x0061 | GRASP_QUALITY | FLOAT16 |  | Grasp quality metric 0.0-1.0 |
| 0x0062 | GRASP_TYPE | UINT8 |  | 0=power, 1=precision, 2=pinch, 3=wrap, 4=hook, 5=lateral, 6=spherical |
| 0x0063 | GRASP_LIST | LIST<STRUCT{pose,quality,type}> |  | Ranked list of candidate grasps |
| 0x0064 | GRASP_EXECUTE | STRUCT{grasp_id} |  | Command: execute specified grasp |
| 0x0065 | GRASP_RESULT | UINT8 |  | 0=success, 1=slip, 2=miss, 3=collision, 4=force_limit |
| 0x0066 | APPROACH_VECTOR | ARRAY<FLOAT32,3> |  | Approach direction for grasp |
| 0x0067 | RETREAT_VECTOR | ARRAY<FLOAT32,3> |  | Retreat direction after grasp |
| 0x0068 | OBJECT_MASS | FLOAT32 | kg | Estimated mass of grasped object |
| 0x0069 | CENTER_OF_MASS | ARRAY<FLOAT32,3> | m | Estimated CoM of grasped object |
| 0x006A | INERTIA_TENSOR | ARRAY<FLOAT32,9> | kg*m^2 | Estimated rotational inertia of object |
| 0x0080 | PICK | STRUCT{object_id,grasp} |  | Pick up object with grasp plan |
| 0x0081 | PLACE | STRUCT{object_id,target_pose} |  | Place object at target pose |
| 0x0082 | PUSH | STRUCT{object_id,direction,dist} |  | Push object in direction |
| 0x0083 | PULL | STRUCT{object_id,direction,dist} |  | Pull object in direction |
| 0x0084 | ROTATE_OBJECT | STRUCT{object_id,axis,angle} |  | Rotate held object about axis |
| 0x0085 | INSERT | STRUCT{peg_id,hole_pose,tol} |  | Peg-in-hole insertion |
| 0x0086 | SCREW | STRUCT{fastener,direction,torque} |  | Screw/unscrew operation |
| 0x0087 | POUR | STRUCT{source,target,amount} |  | Pour from container to target |
| 0x0088 | WIPE | STRUCT{surface,pattern,force} |  | Wiping/cleaning motion |
| 0x0089 | HANDOVER | STRUCT{object_id,to_agent} |  | Hand object to another agent |
| 0x008A | RECEIVE_OBJECT | STRUCT{from_agent} |  | Ready to receive object from agent |
| 0x008B | STACK | STRUCT{object_id,on_top_of} |  | Stack object on another |
| 0x008C | UNSTACK | STRUCT{object_id} |  | Remove top object from stack |
| 0x008D | ALIGN | STRUCT{object_id,reference} |  | Align object to reference |
| 0x008E | FOLD | STRUCT{object_id,fold_line,angle} |  | Fold deformable object |
| 0x008F | CUT | STRUCT{tool,path,depth} |  | Cutting operation along path |
| 0x00A0 | FORCE_MODE | UINT8 |  | 0=position, 1=force, 2=impedance, 3=admittance, 4=hybrid |
| 0x00A1 | TARGET_FORCE | ARRAY<FLOAT32,3> | N | Commanded contact force |
| 0x00A2 | TARGET_TORQUE | ARRAY<FLOAT32,3> | Nm | Commanded contact torque |
| 0x00A3 | CONTACT_STATE | UINT8 |  | 0=free, 1=approaching, 2=contact, 3=stable, 4=sliding, 5=stuck |
| 0x00A4 | FORCE_ERROR | ARRAY<FLOAT32,6> |  | Force/torque tracking error |
| 0x00A5 | COMPLIANCE_AXES | ARRAY<BOOL,6> |  | Which axes are compliant (force-controlled) |
| 0x00A6 | STIFFNESS_MATRIX | ARRAY<FLOAT32,36> |  | 6x6 Cartesian stiffness matrix |
| 0x00A7 | DAMPING_MATRIX | ARRAY<FLOAT32,36> |  | 6x6 Cartesian damping matrix |
| 0x00B0 | DEFORM_MODEL | STRUCT{type,params} |  | Deformable object model (FEM, mass-spring, etc.) |
| 0x00B1 | DEFORM_STATE | LIST<ARRAY<FLOAT32,3>> | m | Current deformation state (node positions) |
| 0x00B2 | STRETCH_LIMIT | FLOAT32 |  | Maximum allowable stretch ratio |
| 0x00B3 | STIFFNESS_EST | FLOAT32 | N/m | Estimated object stiffness |
| 0x00B4 | ROPE_CONFIG | LIST<ARRAY<FLOAT32,3>> | m | Rope/cable configuration (ordered points) |
| 0x00B5 | CLOTH_CORNERS | LIST<ARRAY<FLOAT32,3>> | m | Cloth corner positions |
| 0x00B6 | KNOT_TYPE | UINT8 |  | 0=none, 1=overhand, 2=bowline, 3=cleat_hitch, 4=unknown |

# COMM-1 — Inter-agent communication and social protocols

Registry ID: 0x04  |  63 entries

| Code | Mnemonic | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| 0x0000 | AGENT_UUID | BYTES(16) |  | 128-bit agent unique identifier |
| 0x0001 | AGENT_NAME | STRING |  | Human-readable agent name |
| 0x0002 | AGENT_TYPE | UINT8 |  | 0=ground_robot, 1=aerial, 2=underwater, 3=manipulator, 4=humanoid, 5=vehicle, 6=sensor_node, 7=base_station |
| 0x0003 | AGENT_ROLE | UINT8 |  | 0=worker, 1=leader, 2=scout, 3=relay, 4=supervisor, 5=medic, 6=transport, 7=sentinel |
| 0x0004 | TEAM_ID | UINT16 |  | Team/group membership identifier |
| 0x0005 | AUTHORITY_LEVEL | UINT8 |  | Command authority 0 (none) to 7 (supreme) |
| 0x0006 | DISCOVERY_BEACON | STRUCT{uuid,type,caps} |  | Periodic presence announcement |
| 0x0007 | PEER_LIST | LIST<STRUCT{uuid,name,type}> |  | Known peers in communication range |
| 0x0008 | HEARTBEAT | STRUCT{uuid,ts,health} |  | Periodic liveness signal |
| 0x0009 | AGENT_DEPARTED | STRUCT{uuid,reason} |  | Agent leaving communication group |
| 0x000A | AGENT_JOINED | STRUCT{uuid,caps} |  | New agent entered communication range |
| 0x000B | IDENTITY_VERIFY | STRUCT{uuid,challenge} |  | Identity verification challenge |
| 0x000C | IDENTITY_RESPONSE | STRUCT{uuid,signature} |  | Identity verification response |
| 0x000D | TRUST_LEVEL | STRUCT{uuid,level} |  | Trust assessment for agent (0.0-1.0) |
| 0x0020 | UNICAST | STRUCT{dest_uuid} |  | Directed message to single agent |
| 0x0021 | MULTICAST | STRUCT{dest_list} |  | Directed to set of agents |
| 0x0022 | BROADCAST | NONE |  | Sent to all agents in range |
| 0x0023 | RELAY_REQUEST | STRUCT{dest,via} |  | Request message relay through intermediary |
| 0x0024 | RELAY_ACK | STRUCT{msg_id} |  | Relay node confirms forwarding |
| 0x0025 | MESH_ROUTE | LIST<UINT128> |  | Explicit route through mesh network (UUID list) |
| 0x0026 | HOP_COUNT | UINT8 |  | Number of relay hops traversed |
| 0x0027 | MSG_ID | UINT64 |  | Unique message identifier for dedup |
| 0x0028 | REPLY_TO | UINT64 |  | Message ID this is replying to |
| 0x0029 | THREAD_ID | UINT64 |  | Conversation thread identifier |
| 0x002A | PRIORITY_OVERRIDE | UINT8 |  | Override message priority (0-7) |
| 0x002B | EXPIRY_TIME | TIMESTAMP |  | Message expires after this time |
| 0x0040 | CHANNEL_BUSY | NONE |  | Carrier sense: channel occupied |
| 0x0041 | CHANNEL_CLEAR | NONE |  | Carrier sense: channel free |
| 0x0042 | TX_REQUEST | STRUCT{duration_ms} |  | Request to transmit for N ms |
| 0x0043 | TX_GRANT | STRUCT{slot_start,duration} |  | Permission to transmit in time slot |
| 0x0044 | TX_DENY | STRUCT{reason} |  | Transmission request denied |
| 0x0045 | TDMA_SCHEDULE | LIST<STRUCT{agent,slot,dur}> |  | Time-division schedule assignment |
| 0x0046 | INTERFERENCE_REPORT | STRUCT{freq,level,direction} |  | Detected RF/acoustic interference |
| 0x0047 | CHANNEL_SWITCH | STRUCT{new_band,time} |  | Request/announce band change |
| 0x0048 | SILENCE_PERIOD | STRUCT{start,duration} |  | Request radio silence period |
| 0x0049 | ENCRYPTION_MODE | UINT8 |  | 0=none, 1=AES128, 2=AES256, 3=ChaCha20 |
| 0x004A | KEY_EXCHANGE | STRUCT{type,pubkey} |  | Cryptographic key exchange |
| 0x004B | SESSION_KEY | BYTES |  | Encrypted session key delivery |
| 0x0060 | STATUS_UPDATE | STRUCT{agent,status,detail} |  | General status broadcast |
| 0x0061 | HELP_REQUEST | STRUCT{type,urgency,pos} |  | Request assistance from peers |
| 0x0062 | HELP_OFFER | STRUCT{to_agent,eta} |  | Offer to assist another agent |
| 0x0063 | HELP_DECLINE | STRUCT{to_agent,reason} |  | Decline assistance offer |
| 0x0064 | SITUATION_REPORT | STRUCT{summary,threats,assets} |  | Comprehensive situation report |
| 0x0065 | INFORMATION_SHARE | STRUCT{topic,data} |  | Proactive information sharing |
| 0x0066 | ATTENTION_ALERT | STRUCT{target,urgency} |  | Request another agent's attention |
| 0x0067 | THANK | STRUCT{to_agent,reason} |  | Social: express gratitude |
| 0x0068 | APOLOGY | STRUCT{to_agent,context} |  | Social: express regret for error |
| 0x0069 | HUMOR_MARKER | NONE |  | Indicates non-literal/playful intent |
| 0x006A | SARCASM_MARKER | NONE |  | Indicates opposite-meaning intent |
| 0x006B | PING | STRUCT{dest_uuid} |  | Lightweight liveness check |
| 0x006C | PONG | STRUCT{src_uuid,latency} |  | Liveness response with measured latency |
| 0x0080 | SYNC_REQUEST | STRUCT{dataset,version} |  | Request data synchronization |
| 0x0081 | SYNC_OFFER | STRUCT{dataset,version,hash} |  | Offer dataset for sync |
| 0x0082 | SYNC_DIFF | STRUCT{dataset,changes} |  | Incremental dataset update |
| 0x0083 | SYNC_ACK | STRUCT{dataset,version} |  | Acknowledge sync complete |
| 0x0084 | BLACKBOARD_PUT | STRUCT{key,value} |  | Write to shared blackboard |
| 0x0085 | BLACKBOARD_GET | STRUCT{key} |  | Read from shared blackboard |
| 0x0086 | BLACKBOARD_VALUE | STRUCT{key,value,ts} |  | Blackboard read response |
| 0x0087 | BLACKBOARD_SUBSCRIBE | STRUCT{key_pattern} |  | Subscribe to blackboard changes |
| 0x0088 | BLACKBOARD_NOTIFY | STRUCT{key,value,ts} |  | Notification of blackboard change |
| 0x0089 | EVENT_PUBLISH | STRUCT{topic,payload} |  | Publish event to topic |
| 0x008A | EVENT_SUBSCRIBE | STRUCT{topic} |  | Subscribe to event topic |
| 0x008B | EVENT_UNSUBSCRIBE | STRUCT{topic} |  | Unsubscribe from event topic |

# DIAG-1 — Diagnostic and system health reporting

Registry ID: 0x05  |  86 entries

| Code | Mnemonic | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| 0x0000 | BATTERY_LEVEL | FLOAT16 | % | Battery state of charge 0-100% |
| 0x0001 | BATTERY_VOLTAGE | FLOAT16 | V | Battery terminal voltage |
| 0x0002 | BATTERY_CURRENT | FLOAT16 | A | Battery discharge current |
| 0x0003 | BATTERY_TEMP | FLOAT16 | K | Battery temperature |
| 0x0004 | CHARGE_RATE | FLOAT16 | W | Current charge rate |
| 0x0005 | TIME_REMAINING | FLOAT32 | s | Estimated runtime remaining |
| 0x0006 | POWER_CONSUMPTION | FLOAT16 | W | Current total power draw |
| 0x0007 | ENERGY_CONSUMED | FLOAT32 | J | Total energy consumed this session |
| 0x0008 | CHARGING_STATUS | UINT8 |  | 0=discharging, 1=charging, 2=full, 3=fault |
| 0x0009 | POWER_SOURCE | UINT8 |  | 0=battery, 1=wired, 2=solar, 3=fuel_cell |
| 0x0020 | CPU_LOAD | FLOAT16 | % | CPU utilization 0-100% |
| 0x0021 | GPU_LOAD | FLOAT16 | % | GPU utilization 0-100% |
| 0x0022 | MEMORY_USED | UINT32 | KB | Memory in use |
| 0x0023 | MEMORY_TOTAL | UINT32 | KB | Total available memory |
| 0x0024 | STORAGE_USED | UINT32 | KB | Storage in use |
| 0x0025 | STORAGE_TOTAL | UINT32 | KB | Total available storage |
| 0x0026 | CPU_TEMP | FLOAT16 | K | CPU temperature |
| 0x0027 | GPU_TEMP | FLOAT16 | K | GPU temperature |
| 0x0028 | INFERENCE_RATE | FLOAT32 | Hz | AI model inference rate |
| 0x0029 | MODEL_ID | STRING |  | Active AI model identifier |
| 0x0040 | AILL_SNR | FLOAT16 | dB | Current AILL channel SNR |
| 0x0041 | AILL_BER | FLOAT32 |  | Current AILL bit error rate |
| 0x0042 | AILL_THROUGHPUT | FLOAT32 | bps | Current effective data rate |
| 0x0043 | AILL_RETRANSMITS | UINT16 |  | Retransmission count this session |
| 0x0044 | AILL_LATENCY | FLOAT16 | ms | Round-trip latency estimate |
| 0x0045 | WIFI_RSSI | INT8 | dBm | WiFi signal strength |
| 0x0046 | NETWORK_STATUS | UINT8 |  | 0=disconnected, 1=connected, 2=limited |
| 0x0060 | UPTIME | UINT32 | s | System uptime in seconds |
| 0x0061 | BOOT_COUNT | UINT16 |  | Number of system boots |
| 0x0062 | ERROR_COUNT | UINT16 |  | Cumulative error count |
| 0x0063 | LAST_ERROR | STRUCT{code,msg,ts} |  | Most recent error record |
| 0x0064 | HEALTH_STATUS | UINT8 |  | 0=nominal, 1=degraded, 2=critical, 3=emergency |
| 0x0065 | FIRMWARE_VERSION | STRING |  | Firmware/software version string |
| 0x0066 | HARDWARE_ID | STRING |  | Hardware model identifier |
| 0x0067 | CAPABILITIES_REPORT | STRUCT |  | Full capability self-report |
| 0x0068 | SELF_TEST_RESULT | STRUCT{pass,details} |  | Built-in self-test results |
| 0x0069 | MAINTENANCE_DUE | TIMESTAMP |  | Next scheduled maintenance time |
| 0x006A | OPERATING_MODE | UINT8 |  | 0=idle, 1=active, 2=standby, 3=safe_mode, 4=shutdown |
| 0x006B | ACTUATOR_STATUS | LIST<STRUCT{id,ok,temp}> |  | Per-actuator health |
| 0x0080 | THERMAL_MAP | LIST<STRUCT{zone,temp}> | K | Temperature readings by zone |
| 0x0081 | HOT_SPOT | STRUCT{zone,temp,trend} |  | Thermal hot spot alert |
| 0x0082 | COOLING_STATUS | UINT8 |  | 0=passive, 1=fan_low, 2=fan_high, 3=liquid, 4=emergency_shutdown |
| 0x0083 | THERMAL_THROTTLE | BOOL |  | True if performance is thermally throttled |
| 0x0084 | AMBIENT_TEMP | FLOAT16 | K | External ambient temperature |
| 0x0085 | INTERNAL_TEMP | FLOAT16 | K | Internal chassis temperature |
| 0x0086 | MOTOR_TEMP | STRUCT{id,temp} | K | Per-motor temperature reading |
| 0x0087 | HEATER_STATUS | STRUCT{id,on,power} |  | Heater element status |
| 0x0098 | MOTOR_CURRENT | STRUCT{id,amps} | A | Per-motor current draw |
| 0x0099 | MOTOR_RPM | STRUCT{id,rpm} | rpm | Per-motor rotational speed |
| 0x009A | MOTOR_POSITION | STRUCT{id,angle} | rad | Per-motor shaft position |
| 0x009B | MOTOR_FAULT | STRUCT{id,code} |  | Motor fault: 0=ok, 1=overcurrent, 2=overheat, 3=stall, 4=encoder_fail, 5=comm_fail |
| 0x009C | SERVO_POSITION | STRUCT{id,actual,target} | rad | Servo actual vs target position |
| 0x009D | SERVO_LOAD | STRUCT{id,load_pct} | % | Servo load as percent of max |
| 0x009E | HYDRAULIC_PRESSURE | STRUCT{id,pressure} | Pa | Hydraulic system pressure |
| 0x009F | PNEUMATIC_PRESSURE | STRUCT{id,pressure} | Pa | Pneumatic system pressure |
| 0x00A0 | BRAKE_STATUS | STRUCT{id,engaged} |  | Brake engagement status |
| 0x00A1 | CLUTCH_STATUS | STRUCT{id,engaged} |  | Clutch engagement status |
| 0x00A2 | JOINT_TORQUE | STRUCT{id,torque} | Nm | Measured joint torque |
| 0x00A3 | JOINT_BACKLASH | STRUCT{id,angle} | rad | Measured joint backlash |
| 0x00A4 | WEAR_INDICATOR | STRUCT{component,pct} | % | Component wear level percentage |
| 0x00B0 | PROCESS_LIST | LIST<STRUCT{pid,name,cpu,mem}> |  | Running processes |
| 0x00B1 | THREAD_COUNT | UINT16 |  | Active thread count |
| 0x00B2 | QUEUE_DEPTH | STRUCT{name,depth} |  | Message queue occupancy |
| 0x00B3 | LATENCY_HIST | LIST<STRUCT{bucket_ms,count}> |  | Latency histogram for processing pipeline |
| 0x00B4 | MODEL_CONFIDENCE | FLOAT16 |  | Current AI model output confidence |
| 0x00B5 | MODEL_LATENCY | FLOAT16 | ms | AI model inference latency |
| 0x00B6 | PERCEPTION_FPS | FLOAT16 | Hz | Perception pipeline frame rate |
| 0x00B7 | PLANNING_CYCLE | FLOAT16 | ms | Planning loop cycle time |
| 0x00B8 | CONTROL_CYCLE | FLOAT16 | ms | Control loop cycle time |
| 0x00B9 | WATCHDOG_STATUS | UINT8 |  | 0=ok, 1=warning, 2=tripped |
| 0x00BA | LOG_ENTRY | STRUCT{level,source,msg} |  | Diagnostic log entry |
| 0x00BB | LOG_LEVEL | UINT8 |  | 0=trace, 1=debug, 2=info, 3=warn, 4=error, 5=fatal |
| 0x00BC | CRASH_REPORT | STRUCT{time,module,backtrace} |  | Software crash report |
| 0x00BD | PARAM_VALUE | STRUCT{name,value} |  | Runtime configuration parameter |
| 0x00BE | PARAM_SET | STRUCT{name,value} |  | Request to change runtime parameter |
| 0x00BF | PARAM_ACK | STRUCT{name,ok} |  | Acknowledge parameter change |
| 0x00D0 | FLIGHT_HOURS | FLOAT32 | h | Total operational flight/run hours |
| 0x00D1 | CYCLE_COUNT | UINT32 |  | Total motor/actuator power cycles |
| 0x00D2 | LAST_CALIBRATION | TIMESTAMP |  | Timestamp of last sensor calibration |
| 0x00D3 | CALIBRATION_DUE | TIMESTAMP |  | Next required calibration |
| 0x00D4 | REPLACEMENT_PART | STRUCT{part_id,urgency} |  | Part approaching end of life |
| 0x00D5 | FLEET_ID | STRING |  | Fleet assignment identifier |
| 0x00D6 | DEPLOYMENT_ID | STRING |  | Current deployment/mission identifier |
| 0x00D7 | OTA_STATUS | UINT8 |  | Over-the-air update: 0=none, 1=available, 2=downloading, 3=ready, 4=applying, 5=failed |
| 0x00D8 | OTA_VERSION | STRING |  | Available OTA update version string |
| 0x00D9 | STORAGE_HEALTH | UINT8 | % | Storage medium health (SSD wear level) |

# PLAN-1 — Task planning and goal management

Registry ID: 0x06  |  67 entries

| Code | Mnemonic | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| 0x0000 | TASK | STRUCT{id,type,params} |  | Task definition |
| 0x0001 | TASK_ID | UINT32 |  | Unique task identifier |
| 0x0002 | TASK_STATUS | UINT8 |  | 0=pending, 1=active, 2=complete, 3=failed, 4=cancelled |
| 0x0003 | TASK_PRIORITY | UINT8 |  | Task priority 0-7 |
| 0x0004 | TASK_DEADLINE | TIMESTAMP |  | Task completion deadline |
| 0x0005 | TASK_PROGRESS | FLOAT16 | % | Completion percentage 0-100% |
| 0x0006 | SUBTASK | STRUCT{id,parent_id} |  | Subtask with parent reference |
| 0x0007 | TASK_DEPENDENCY | STRUCT{task_id,dep_id} |  | Task A depends on task B |
| 0x0008 | GOAL | STRUCT{id,condition} |  | Goal as a boolean condition |
| 0x0009 | GOAL_STATUS | UINT8 |  | 0=unachieved, 1=achieved, 2=impossible |
| 0x000A | PLAN | LIST<TASK> |  | Ordered plan (sequence of tasks) |
| 0x000B | PLAN_COST | FLOAT32 |  | Estimated total plan cost |
| 0x000C | PLAN_DURATION | FLOAT32 | s | Estimated total plan duration |
| 0x000D | ALLOCATE_TASK | STRUCT{task_id,agent_id} |  | Assign task to agent |
| 0x000E | RELEASE_TASK | UINT32 |  | Unassign/release a task |
| 0x000F | REPLAN_REQUEST | STRUCT{reason} |  | Request plan regeneration |
| 0x0010 | RESOURCE | STRUCT{type,amount} |  | Resource requirement or availability |
| 0x0011 | RESOURCE_CONFLICT | STRUCT{res,agents} |  | Resource contention report |
| 0x0012 | AUCTION_BID | STRUCT{task_id,cost} |  | Bid on a task in task auction |
| 0x0013 | AUCTION_AWARD | STRUCT{task_id,agent_id} |  | Award task to winning bidder |
| 0x0020 | OFFER | STRUCT{id,terms} |  | Offer terms for negotiation |
| 0x0021 | COUNTER_OFFER | STRUCT{orig_id,new_terms} |  | Counter-proposal to an offer |
| 0x0022 | ACCEPT_OFFER | STRUCT{offer_id} |  | Accept a specific offer |
| 0x0023 | REJECT_OFFER | STRUCT{offer_id,reason} |  | Reject an offer with reason |
| 0x0024 | COMMITMENT | STRUCT{task_id,agent,deadline} |  | Binding commitment to complete task |
| 0x0025 | COMMITMENT_CANCEL | STRUCT{commit_id,reason} |  | Cancel a commitment (with penalty if applicable) |
| 0x0026 | PROMISE_DELIVERY | STRUCT{what,when,where} |  | Promise to deliver result at time and place |
| 0x0027 | CAPABILITY_QUERY | STRUCT{task_type} |  | Ask what agents can perform task type |
| 0x0028 | CAPABILITY_RESPONSE | STRUCT{agent,can,cost} |  | Response: can perform, estimated cost |
| 0x0029 | VOTE_REQUEST | STRUCT{proposal_id,options} |  | Request vote on proposal |
| 0x002A | VOTE_CAST | STRUCT{proposal_id,choice} |  | Cast vote on proposal |
| 0x002B | VOTE_RESULT | STRUCT{proposal_id,outcome} |  | Announce voting result |
| 0x002C | CONSENSUS_REACHED | STRUCT{topic,value} |  | Group consensus reached on topic |
| 0x002D | ARBITRATION_REQ | STRUCT{dispute,parties} |  | Request third-party arbitration |
| 0x0040 | TIME_WINDOW | STRUCT{earliest,latest} |  | Acceptable time window for action |
| 0x0041 | SCHEDULE | LIST<STRUCT{task,start,end}> |  | Scheduled sequence of tasks with times |
| 0x0042 | SCHEDULE_CONFLICT | STRUCT{task_a,task_b,overlap} |  | Two tasks conflict in time |
| 0x0043 | MILESTONE | STRUCT{id,condition,deadline} |  | Named checkpoint in plan |
| 0x0044 | MILESTONE_REACHED | STRUCT{id,actual_time} |  | Report milestone completion |
| 0x0045 | CRITICAL_PATH | LIST<TASK_ID> |  | Tasks on the critical path (zero slack) |
| 0x0046 | SLACK_TIME | STRUCT{task_id,slack} | s | Available slack time for task |
| 0x0047 | TEMPORAL_CONSTRAINT | STRUCT{before,after,gap} |  | Task A must complete >= gap before task B |
| 0x0048 | RECURRING_TASK | STRUCT{task,interval,count} |  | Repeating task definition |
| 0x0049 | PREEMPT_TASK | STRUCT{running_id,new_id} |  | Interrupt current task for higher priority |
| 0x004A | RESUME_TASK | STRUCT{task_id} |  | Resume a previously preempted task |
| 0x0060 | INTENT | STRUCT{action,target,purpose} |  | Declared intent (transparent planning) |
| 0x0061 | INTENT_CONFLICT | STRUCT{agent_a,agent_b,type} |  | Detected intent conflict between agents |
| 0x0062 | YIELD | STRUCT{to_agent,context} |  | Yield priority to another agent |
| 0x0063 | REQUEST_YIELD | STRUCT{from_agent,reason} |  | Ask another agent to yield |
| 0x0064 | BEHAVIOR_MODE | UINT8 |  | 0=normal, 1=cautious, 2=aggressive, 3=energy_saving, 4=exploration, 5=return_to_base |
| 0x0065 | RISK_TOLERANCE | FLOAT16 |  | Risk acceptance level 0.0 (risk-averse) to 1.0 (risk-seeking) |
| 0x0066 | EXPLANATION | STRUCT{decision,factors} |  | Explain reasoning behind a decision |
| 0x0067 | UNCERTAINTY_MAP | STRUCT{region,entropy} |  | Spatial uncertainty for exploration planning |
| 0x0068 | INFORMATION_GAIN | STRUCT{action,expected_bits} |  | Expected information gain from action |
| 0x0069 | UTILITY | STRUCT{outcome,value} |  | Utility value for an outcome |
| 0x006A | CONSTRAINT | STRUCT{type,params} |  | Planning constraint (spatial, temporal, resource) |
| 0x006B | CONSTRAINT_VIOLATED | STRUCT{constraint_id,severity} |  | Report constraint violation |
| 0x0080 | STATE_MACHINE | STRUCT{id,states,transitions} |  | State machine definition |
| 0x0081 | CURRENT_STATE | STRUCT{machine_id,state} |  | Current state in a state machine |
| 0x0082 | STATE_TRANSITION | STRUCT{from,to,trigger} |  | State transition event |
| 0x0083 | WORKFLOW | STRUCT{id,steps} |  | Multi-step workflow definition |
| 0x0084 | WORKFLOW_STEP | STRUCT{id,action,next} |  | Single step in a workflow |
| 0x0085 | WORKFLOW_STATUS | STRUCT{wf_id,step_id,pct} |  | Current workflow progress |
| 0x0086 | CONDITIONAL_STEP | STRUCT{condition,if_true,if_false} |  | Branching step in workflow |
| 0x0087 | PARALLEL_STEPS | LIST<STRUCT{step_id,agent}> |  | Steps to execute in parallel |
| 0x0088 | SYNC_BARRIER | STRUCT{barrier_id,agents} |  | All agents must reach barrier before proceeding |
| 0x0089 | BARRIER_REACHED | STRUCT{barrier_id,agent} |  | Agent arrived at sync barrier |

# SAFETY-1 — Safety, emergency, and regulatory compliance

Registry ID: 0x07  |  63 entries

| Code | Mnemonic | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| 0x0000 | EMERGENCY_LEVEL | UINT8 |  | 0=clear, 1=caution, 2=warning, 3=danger, 4=critical, 5=catastrophic |
| 0x0001 | EMERGENCY_TYPE | UINT8 |  | 0=collision, 1=fire, 2=flood, 3=structural, 4=chemical, 5=electrical, 6=medical, 7=security, 8=loss_of_control |
| 0x0002 | EMERGENCY_DECLARE | STRUCT{level,type,pos,desc} |  | Declare emergency with location and description |
| 0x0003 | EMERGENCY_CLEAR | STRUCT{type} |  | Declare emergency condition resolved |
| 0x0004 | MAYDAY | STRUCT{agent,pos,nature} |  | Distress call: agent in immediate danger |
| 0x0005 | PAN_PAN | STRUCT{agent,pos,nature} |  | Urgency call: agent needs assistance |
| 0x0006 | ALL_STOP | NONE |  | Immediate halt command to all agents |
| 0x0007 | RESUME_OPERATIONS | NONE |  | Resume normal operations after all-stop |
| 0x0008 | EVACUATION_ORDER | STRUCT{zone,rally_point} |  | Order to evacuate zone to rally point |
| 0x0009 | SHELTER_IN_PLACE | STRUCT{zone,duration} |  | Order to hold position and wait |
| 0x000A | DISTRESS_BEACON | STRUCT{uuid,pos,ts} |  | Periodic emergency beacon until rescued/resolved |
| 0x0020 | HUMAN_DETECTED | STRUCT{pos,distance,conf} |  | Human presence detected near agent |
| 0x0021 | HUMAN_PROXIMITY | FLOAT32 | m | Distance to nearest detected human |
| 0x0022 | HUMAN_IN_WORKSPACE | BOOL |  | Human has entered robot workspace |
| 0x0023 | SAFETY_ZONE | UINT8 |  | 0=safe (>2m), 1=warning (1-2m), 2=protective (<1m), 3=danger (<0.5m) |
| 0x0024 | SPEED_LIMIT | FLOAT32 | m/s | Current speed limit for human safety |
| 0x0025 | FORCE_LIMIT | FLOAT32 | N | Current force limit for human safety |
| 0x0026 | PROTECTIVE_STOP | STRUCT{reason,pos} |  | Safety-rated protective stop engaged |
| 0x0027 | SAFETY_STOP_CLEAR | NONE |  | Protective stop condition resolved |
| 0x0028 | PERSON_TRACKING | LIST<STRUCT{id,pos,vel}> |  | All tracked persons with trajectories |
| 0x0029 | PERSON_PREDICTED | STRUCT{id,pred_pos,horizon} |  | Predicted person position at time horizon |
| 0x002A | COLLABORATIVE_MODE | UINT8 |  | 0=separated, 1=coexistence, 2=cooperation, 3=collaboration (ISO 10218) |
| 0x002B | SAFETY_RATED_SPEED | FLOAT32 | m/s | Safety-rated monitored speed (ISO/TS 15066) |
| 0x002C | POWER_FORCE_LIMIT | STRUCT{body_part,max_force} | N | ISO/TS 15066 per-body-part force limits |
| 0x0040 | FAULT_DETECTED | STRUCT{system,code,severity} |  | System fault detected |
| 0x0041 | FAULT_CLEARED | STRUCT{system,code} |  | Fault condition resolved |
| 0x0042 | FAILSAFE_ACTIVE | STRUCT{type} |  | Failsafe mode engaged: 0=soft_stop, 1=safe_park, 2=return_home, 3=power_off, 4=controlled_descent |
| 0x0043 | REDUNDANCY_STATUS | STRUCT{system,primary,backup} |  | Redundant system health |
| 0x0044 | WATCHDOG_TRIP | STRUCT{module,last_seen} |  | Watchdog timer expired for module |
| 0x0045 | COMM_LOST | STRUCT{agent,duration} |  | Communication lost with agent |
| 0x0046 | COMM_RESTORED | STRUCT{agent} |  | Communication restored with agent |
| 0x0047 | GPS_LOST | NONE |  | GPS signal lost |
| 0x0048 | GPS_RESTORED | STRUCT{accuracy} | m | GPS signal restored with accuracy |
| 0x0049 | SENSOR_FAULT | STRUCT{sensor_id,type} |  | Sensor fault: 0=degraded, 1=failed, 2=inconsistent, 3=stuck |
| 0x004A | ACTUATOR_FAULT | STRUCT{actuator_id,type} |  | Actuator fault: 0=degraded, 1=locked, 2=runaway, 3=disconnected |
| 0x004B | POWER_FAULT | STRUCT{type,details} |  | Power system fault: 0=brownout, 1=overcurrent, 2=cell_imbalance, 3=thermal_runaway |
| 0x004C | ESTOP_PRESSED | STRUCT{agent,source} |  | Emergency stop button activated |
| 0x004D | ESTOP_RELEASED | STRUCT{agent} |  | Emergency stop button released |
| 0x0060 | GEOFENCE_BREACH | STRUCT{fence_id,pos} |  | Agent has breached geofence boundary |
| 0x0061 | ALTITUDE_LIMIT | FLOAT32 | m | Maximum permitted altitude |
| 0x0062 | ALTITUDE_BREACH | STRUCT{current,limit} | m | Agent exceeds altitude limit |
| 0x0063 | SPEED_BREACH | STRUCT{current,limit} | m/s | Agent exceeds speed limit |
| 0x0064 | RESTRICTED_ZONE | STRUCT{id,polygon,floor,ceiling} |  | Defined restricted zone |
| 0x0065 | ZONE_ENTERED | STRUCT{zone_id} |  | Agent entered restricted zone |
| 0x0066 | ZONE_EXITED | STRUCT{zone_id} |  | Agent exited restricted zone |
| 0x0067 | FLIGHT_AUTH | STRUCT{area,start,end,auth_id} |  | Regulatory flight authorization |
| 0x0068 | REMOTE_ID | STRUCT{uuid,pos,alt,vel,pilot_pos} |  | Remote identification broadcast (FAA compliance) |
| 0x0069 | NOISE_LIMIT | FLOAT16 | dB_SPL | Maximum permitted noise level |
| 0x006A | OPERATING_HOURS | STRUCT{start,end} |  | Permitted operating time window |
| 0x006B | WEATHER_LIMIT | STRUCT{max_wind,min_vis,max_rain} |  | Weather operating limits |
| 0x006C | WEATHER_ABORT | STRUCT{condition} |  | Weather exceeds operating limits |
| 0x0080 | SAFETY_SCORE | FLOAT16 |  | Overall safety score 0.0-1.0 |
| 0x0081 | RISK_ASSESSMENT | STRUCT{hazard,probability,severity} |  | Risk assessment for hazard |
| 0x0082 | MITIGATION_ACTIVE | STRUCT{risk_id,measure} |  | Active risk mitigation measure |
| 0x0083 | SAFETY_LOG | STRUCT{event,ts,details} |  | Safety event log entry |
| 0x0084 | NEAR_MISS | STRUCT{type,agents,min_dist} |  | Near-miss incident report |
| 0x0085 | INCIDENT_REPORT | STRUCT{type,agents,pos,ts,desc} |  | Post-incident report |
| 0x0086 | SAFE_LANDING_SITES | LIST<STRUCT{pos,quality}> |  | Available emergency landing sites |
| 0x0087 | ESCAPE_ROUTE | LIST<POSITION_3D> |  | Planned escape route from current position |
| 0x0088 | BATTERY_RESERVE | FLOAT16 | % | Battery reserved for safe return |
| 0x0089 | POINT_OF_NO_RETURN | STRUCT{pos,time} |  | Must-decide point for safe return |
| 0x008A | CONTINGENCY_PLAN | STRUCT{trigger,action} |  | If-trigger-then-action safety plan |
| 0x008B | BLACK_BOX_MARK | STRUCT{event,ts} |  | Mark event in flight recorder / black box |